SOFTWARE DESIGN DOCUMENT (SDD)

Project Title

Machine Learning-Based Adaptive Traffic Signal System

Document: Software Design Document
Version: 1.0

1. Introduction

1.1 Purpose

This Software Design Document (SDD) describes the technical design of the Machine Learning-Based Adaptive Traffic Signal System.

The SDD translates the requirements defined in the SRS into a practical system design. It describes the system architecture, software modules, database structure, machine-learning pipeline, adaptive traffic algorithm, Arduino communication, web dashboard, interfaces, and major system interactions.

The system is designed as a prototype for one four-way intersection.

1.2 System Overview

The system uses a camera to capture vehicles approaching an intersection.

The captured video is processed by a machine-learning model to detect vehicles. A tracking mechanism follows vehicles between frames and prevents the same vehicle from being counted repeatedly.

The system then determines which lane has the highest traffic demand.

An adaptive algorithm calculates the appropriate green-light duration, and the resulting command is sent to an Arduino microcontroller.

Arduino controls the prototype traffic lights.

At the same time, a web application displays the current traffic conditions and system status.

The complete process is:

Camera

↓

Video Capture

↓

Machine Learning

↓

Vehicle Detection

↓

Vehicle Tracking

↓

Lane Assignment

↓

Vehicle Counting

↓

Traffic Analysis

↓

Adaptive Decision

↓

Arduino

↓

Traffic Lights

The web dashboard operates alongside this process:

Traffic Processing

│

├──────────────► Database

│

└──────────────► Web Dashboard

2. System Architecture

2.1 High-Level Architecture

The system will use a layered architecture.

┌───────────────────────────────────────────────┐

│                 PRESENTATION                   │

│                                               │

│              Web Dashboard                    │

└───────────────────────┬───────────────────────┘

│

▼

┌───────────────────────────────────────────────┐

│                  APPLICATION                  │

│                                               │

│ Traffic Analysis │ Adaptive Controller       │

│ System Management │ API                      │

└───────────────┬───────────────────┬───────────┘

│                   │

▼                   ▼

┌────────────────────────┐   ┌──────────────────┐

│   MACHINE LEARNING     │   │ HARDWARE LAYER   │

│                        │   │                  │

│ Detection              │   │ Arduino          │

│ Tracking               │   │ Traffic Lights   │

│ Lane Counting          │   │ Serial           │

└────────────┬───────────┘   └──────────────────┘

│

▼

┌─────────────┐

│   CAMERA    │

└─────────────┘

│

▼

┌─────────────┐

│  DATABASE   │

└─────────────┘

2.2 Main System Components

The system will contain the following major components:

- Camera Module

- Machine Learning Detection Module

- Vehicle Tracking Module

- Lane Counting Module

- Traffic Analysis Module

- Adaptive Signal Controller

- Arduino Communication Module

- Database Module

- Backend/API Module

- Web Dashboard Module

3. Component Design

3.1 Camera Module

Purpose

The Camera Module captures video of vehicles approaching the intersection.

Input

Camera

Output

Video Frames

Process

Camera

↓

Capture Frame

↓

Resize/Preprocess

↓

Send to ML Module

Responsibilities

- Initialize camera.

- Capture frames.

- Check camera availability.

- Provide frames to the ML module.

- Report camera errors.

3.2 Machine Learning Detection Module

Purpose

This module detects vehicles in each camera frame.

A pre-trained object-detection model will be used rather than training a large machine-learning model from scratch.

Input

Video Frame

Output

Vehicle

Bounding Box

Confidence

Class

Example:

Vehicle:

Class = Car

Confidence = 0.91

Position = (x, y)

Processing

Frame

↓

ML Model

↓

Object Detection

↓

Filter Vehicle Classes

↓

Valid Vehicle Detections

3.3 Vehicle Tracking Module

Purpose

The tracking module prevents the same vehicle from being counted repeatedly.

Without tracking:

Frame 1 → Car

Frame 2 → Car

Frame 3 → Car

Frame 4 → Car

could incorrectly become:

4 vehicles

The tracking module instead determines:

Frame 1 → Vehicle 001

Frame 2 → Vehicle 001

Frame 3 → Vehicle 001

Frame 4 → Vehicle 001

Therefore:

Actual Count = 1 vehicle

Input

Vehicle detections.

Output

Tracked vehicles with temporary IDs.

ID 001

ID 002

ID 003

3.4 Lane Assignment Module

The camera view will be divided into predefined regions representing traffic approaches.

Example:

NORTH

↑

│

┌─────────┼─────────┐

│         │         │

│         │         │

WEST ←─┼─────────┼─────────┼─→ EAST

│         │         │

│         │         │

└─────────┼─────────┘

│

↓

SOUTH

The system will use the position of a tracked vehicle to determine its lane.

For the prototype, lane regions can be configured manually.

3.5 Vehicle Counting Module

The Vehicle Counting Module calculates the number of vehicles associated with each lane.

Example:

North = 8

South = 15

East  = 4

West  = 7

The module receives tracked vehicles and determines their lane.

Processing

Tracked Vehicle

↓

Determine Position

↓

Determine Lane

↓

Update Count

3.6 Traffic Analysis Module

The Traffic Analysis Module determines the traffic level of each lane.

Example configuration:

0–5 vehicles    = LOW

6–10 vehicles   = MEDIUM

11+ vehicles    = HIGH

Therefore:

North = 8   → MEDIUM

South = 15  → HIGH

East  = 4   → LOW

West  = 7   → MEDIUM

The thresholds will be configurable.

3.7 Adaptive Signal Controller

This is one of the most important components of the system.

Its responsibility is to determine:

- Which lane receives priority.

- How long the green light should remain active.

Input

Vehicle counts

Traffic levels

Waiting information

Current signal state

Output

Priority Lane

Green Time

3.8 Priority Algorithm

The initial system will use a simple demand-based algorithm.

Suppose:

North = 5

South = 18

East  = 4

West  = 7

The controller compares the values.

MAX(5,18,4,7)

Therefore:

Priority = SOUTH

3.9 Green-Time Algorithm

The initial green-time calculation will use:

Green Time =

Base Time + (Vehicle Count × Additional Time)

Example:

Base Time = 20 seconds

Additional Time = 2 seconds

For South:

Vehicle Count = 18

Green Time =

20 + (18 × 2)

= 56 seconds

The system will then apply the limits:

Minimum Green = 20 seconds

Maximum Green = 60 seconds

Therefore:

Final Green Time = 56 seconds

3.10 Fairness Mechanism

A purely vehicle-count-based system could repeatedly select one lane if it continuously has more vehicles.

To reduce this problem, the system can include a simple waiting mechanism.

Example:

Demand Score =

Vehicle Count + Waiting Bonus

If a lane has been waiting for several cycles, its score increases.

Example:

South:

Vehicles = 10

Waiting Bonus = 5

Score = 15

This allows other lanes to eventually receive service.

This mechanism is intentionally simple and does not require reinforcement learning.

4. Signal Control Design

4.1 Signal States

Each direction will have:

RED

YELLOW

GREEN

The normal sequence will be:

RED

↓

GREEN

↓

YELLOW

↓

RED

4.2 Intersection Signal Logic

For a simple prototype, opposing directions can operate together.

Example:

North + South

↓

GREEN

East + West

↓

RED

Then:

North + South

↓

YELLOW

East + West

↓

RED

Then:

North + South

↓

RED

East + West

↓

GREEN

This prevents conflicting directions from receiving green simultaneously.

5. Arduino Design

5.1 Purpose

Arduino provides the hardware-level control of the prototype traffic lights.

The computer performs the machine-learning and traffic-analysis operations.

Arduino does not run the ML model.

5.2 Arduino Components

The prototype will contain:

- Arduino Uno

- Red LEDs

- Yellow LEDs

- Green LEDs

- Resistors

- Breadboard

- Jumper wires

- USB connection

5.3 Arduino Communication

The computer will communicate with Arduino through USB serial communication.

Example command:

SOUTH,56

Meaning:

Priority Lane = SOUTH

Green Time = 56 seconds

Arduino receives the command and controls the LEDs.

5.4 Arduino Processing

Receive Command

↓

Parse Lane

↓

Parse Duration

↓

Set Other Directions RED

↓

Set Selected Direction GREEN

↓

Wait

↓

Set YELLOW

↓

Set RED

↓

Wait for Next Command

6. Backend Design

The backend will connect the machine-learning system, database, Arduino, and web dashboard.

A lightweight Python backend such as Flask will be used.

Backend Responsibilities

- Receive traffic information.

- Provide API endpoints.

- Store traffic data.

- Retrieve historical data.

- Communicate with Arduino.

- Provide system status.

- Provide data to the dashboard.

6.1 Proposed Backend Structure

backend/

│

├── app.py

│

├── routes/

│   ├── traffic.py

│   ├── signal.py

│   └── system.py

│

├── ml/

│   ├── detector.py

│   ├── tracker.py

│   └── counter.py

│

├── control/

│   ├── analyzer.py

│   └── adaptive_controller.py

│

├── hardware/

│   └── arduino.py

│

├── database/

│   ├── models.py

│   └── database.py

│

└── config.py

This modular structure will make the project easier to maintain.

7. API Design

The backend will expose simple API endpoints.

7.1 Get Current Traffic

GET /api/traffic/current

Example response:

{

"north": 8,

"south": 15,

"east": 4,

"west": 7

}

7.2 Get Current Signal

GET /api/signal/current

Example:

{

"priority_lane": "south",

"state": "green",

"green_time": 48,

"remaining_time": 32

}

7.3 Get System Status

GET /api/system/status

Example:

{

"camera": "connected",

"ml": "running",

"arduino": "connected",

"database": "connected"

}

7.4 Get Traffic History

GET /api/traffic/history

The endpoint will return historical traffic records for the dashboard.

8. Database Design

The system will use a lightweight relational database such as SQLite.

8.1 Entity Relationship Overview

┌────────────────────┐

│  TRAFFIC_RECORD    │

├────────────────────┤

│ id                 │

│ timestamp          │

│ lane               │

│ vehicle_count      │

│ traffic_level      │

└────────────────────┘

│

│

┌────────────────────┐

│   SIGNAL_EVENT     │

├────────────────────┤

│ id                 │

│ timestamp          │

│ priority_lane      │

│ green_time         │

│ signal_state       │

└────────────────────┘

│

│

┌────────────────────┐

│   SYSTEM_EVENT     │

├────────────────────┤

│ id                 │

│ timestamp          │

│ component          │

│ status             │

│ message            │

└────────────────────┘

8.2 Traffic Record Table

traffic_records

--------------------------------

id              INTEGER PK

timestamp       DATETIME

lane            VARCHAR

vehicle_count   INTEGER

traffic_level   VARCHAR

Example:

| ID | Lane | Vehicles | Level |
| --- | --- | --- | --- |
| 1 | North | 8 | Medium |
| 2 | South | 15 | High |
| 3 | East | 4 | Low |
| 4 | West | 7 | Medium |

8.3 Signal Event Table

signal_events

--------------------------------

id              INTEGER PK

timestamp       DATETIME

priority_lane   VARCHAR

green_time      INTEGER

signal_state    VARCHAR

Example:

| Lane | Green Time | State |
| --- | --- | --- |
| South | 48 | Green |
| North | 30 | Green |

8.4 System Event Table

system_events

--------------------------------

id              INTEGER PK

timestamp       DATETIME

component       VARCHAR

status          VARCHAR

message         TEXT

This table will record events such as:

Camera connected

Arduino disconnected

ML model started

Database connected

9. Web Dashboard Design

The web interface will be intentionally simple.

9.1 Main Dashboard

┌──────────────────────────────────────────────────┐

│        MACHINE LEARNING TRAFFIC SYSTEM           │

├──────────────────────────────────────────────────┤

│                                                  │

│ NORTH          SOUTH          EAST        WEST   │

│  8             15             4           7     │

│ MEDIUM         HIGH           LOW        MEDIUM  │

│                                                  │

├──────────────────────────────────────────────────┤

│ PRIORITY LANE: SOUTH                             │

│                                                  │

│ CURRENT SIGNAL: GREEN                           │

│ GREEN TIME: 48 sec                              │

│ REMAINING: 32 sec                               │

│                                                  │

├──────────────────────────────────────────────────┤

│ CAMERA     ✓ CONNECTED                          │

│ ML MODEL   ✓ RUNNING                            │

│ ARDUINO    ✓ CONNECTED                          │

│ DATABASE   ✓ CONNECTED                          │

└──────────────────────────────────────────────────┘

9.2 Dashboard Components

The dashboard will contain:

Traffic Cards

Displays vehicle counts.

Traffic Level

Displays:

LOW

MEDIUM

HIGH

Priority Lane

Displays the lane currently receiving priority.

Signal Status

Displays:

RED

YELLOW

GREEN

Timer

Displays remaining green time.

System Status

Displays component health.

Traffic History

Displays historical traffic information.

10. Data Flow Design

10.1 Complete Data Flow

CAMERA

│

▼

VIDEO FRAMES

│

▼

┌────────────────────┐

│ ML VEHICLE DETECTOR│

└──────────┬─────────┘

│

▼

VEHICLE BOXES

│

▼

┌────────────────────┐

│ VEHICLE TRACKER    │

└──────────┬─────────┘

│

▼

TRACKED VEHICLES

│

▼

┌────────────────────┐

│ LANE ASSIGNMENT    │

└──────────┬─────────┘

│

▼

VEHICLE COUNTS

│

▼

┌────────────────────┐

│ TRAFFIC ANALYZER   │

└──────────┬─────────┘

│

▼

┌────────────────────┐

│ ADAPTIVE CONTROLLER│

└───────┬─────┬──────┘

│     │

│     └─────────────► DATABASE

│

├───────────────────► WEB

│

▼

ARDUINO

│

▼

TRAFFIC LIGHTS

11. Sequence Diagram

11.1 Normal System Operation

Camera       ML       Tracker      Analyzer      Arduino      Dashboard

│           │          │            │             │              │

│ Frame     │          │            │             │              │

├──────────►│          │            │             │              │

│           │ Detect   │            │             │              │

│           ├─────────►│            │             │              │

│           │          │ Track      │             │              │

│           │          ├───────────►│             │              │

│           │          │            │ Analyze     │              │

│           │          │            ├────────────►│              │

│           │          │            │             │ Control      │

│           │          │            │             │ Lights       │

│           │          │            │             │              │

│           │          │            ├────────────────────────────►│

│           │          │            │             │              │

│           │          │            │             │              │

│           │          │            │             │              │

12. Activity Diagram

START

│

▼

Initialize System

│

▼

Connect Camera

│

┌────┴────┐

│ Camera? │

└────┬────┘

Yes│

▼

Capture Frame

│

▼

Detect Vehicles

│

▼

Track Vehicles

│

▼

Assign Lanes

│

▼

Count Vehicles

│

▼

Analyze Traffic

│

▼

Select Priority

│

▼

Calculate Time

│

▼

Send to Arduino

│

▼

Control Signals

│

▼

Store Information

│

▼

Update Dashboard

│

▼

Reassess Traffic

│

└─────────────►

13. Class Design

The software can be organized into the following main classes.

13.1 Vehicle

Vehicle

---------------------

id

vehicle_type

confidence

x

y

lane

timestamp

13.2 CameraManager

CameraManager

---------------------

connect()

capture_frame()

release()

get_status()

13.3 VehicleDetector

VehicleDetector

---------------------

load_model()

detect(frame)

filter_vehicles()

13.4 VehicleTracker

VehicleTracker

---------------------

update(detections)

get_tracks()

remove_old_tracks()

13.5 TrafficAnalyzer

TrafficAnalyzer

---------------------

count_by_lane()

calculate_traffic_level()

calculate_demand()

13.6 AdaptiveController

AdaptiveController

---------------------

select_priority_lane()

calculate_green_time()

apply_limits()

check_fairness()

13.7 ArduinoController

ArduinoController

---------------------

connect()

send_command()

set_signal()

disconnect()

get_status()

13.8 DatabaseManager

DatabaseManager

---------------------

save_traffic()

save_signal_event()

save_system_event()

get_history()

14. Adaptive Algorithm

The proposed algorithm is:

START

│

▼

Get Vehicle Counts

│

▼

Calculate Demand Score

│

▼

Find Highest Score

│

▼

Select Priority Lane

│

▼

Calculate Green Time

│

▼

Apply Minimum Limit

│

▼

Apply Maximum Limit

│

▼

Check Fairness

│

▼

Send Command

│

▼

Control Traffic Light

│

▼

Monitor Traffic

│

└──────────────► REPEAT

15. Pseudocode

START

initialize camera

initialize ML model

initialize tracker

initialize Arduino

initialize database

WHILE system is running:

frame = capture_frame()

detections = detect_vehicles(frame)

tracked_vehicles = track(detections)

lane_counts = count_vehicles(tracked_vehicles)

traffic_levels = classify_traffic(lane_counts)

demand_scores = calculate_demand(

lane_counts,

waiting_information

)

priority_lane = select_highest_score(

demand_scores

)

green_time = calculate_green_time(

lane_counts[priority_lane]

)

green_time = apply_min_max_limits(

green_time

)

send_command_to_arduino(

priority_lane,

green_time

)

save_traffic_data()

save_signal_data()

update_dashboard()

END WHILE

16. Machine Learning Pipeline

The machine-learning component will follow this pipeline:

Camera

↓

Frame

↓

Preprocessing

↓

Object Detection Model

↓

Vehicle Bounding Boxes

↓

Confidence Filtering

↓

Vehicle Tracking

↓

Lane Assignment

↓

Vehicle Count

17. Model Design Considerations

The project should prioritize practical implementation over unnecessary ML complexity.

The system can begin with a suitable pre-trained object-detection model.

The workflow will be:

Pre-trained Model

↓

Run on Traffic Video

↓

Evaluate Vehicle Detection

↓

Tune Confidence Threshold

↓

Integrate Tracking

↓

Integrate Lane Counting

If the selected model performs poorly on the project's specific camera footage, a small dataset can later be collected and used for fine-tuning.

This keeps the project manageable while still making machine learning the central component.

18. Error Handling Design

18.1 Camera Error

Camera unavailable

↓

Log Error

↓

Display Warning

↓

Attempt Reconnection

18.2 Arduino Error

Arduino unavailable

↓

Log Error

↓

Display Status

↓

Attempt Reconnection

18.3 ML Error

ML processing error

↓

Log Error

↓

Notify Operator

↓

Restart/Recover Module

19. Security Design

Although the prototype is not a large public system, basic security will be included.

Authentication

The dashboard can require login credentials.

Password Storage

Passwords should be hashed rather than stored directly.

API Protection

Sensitive operations should require authorization.

Configuration Protection

Only authorized users should change:

- Minimum green time.

- Maximum green time.

- Detection threshold.

- Additional time per vehicle.

20. Performance Design

The system will not necessarily process every camera frame.

For example:

Camera:

30 FPS

Processing:

10 FPS

may be sufficient for the prototype.

Reducing the processing frame rate can decrease computer workload while still providing useful traffic information.

The exact configuration will depend on available hardware.

21. Scalability Design

The initial system is:

1 Camera

↓

1 Intersection

↓

1 Arduino

Future versions could become:

Camera 1 ──┐

Camera 2 ──┼──► Traffic Management Server

Camera 3 ──┤

Camera 4 ──┘

│

┌───────┼────────┐

▼       ▼        ▼

Arduino  Arduino  Arduino

Junction  Junction Junction

The current architecture should therefore separate traffic processing from hardware control so that additional intersections can be added later.

22. Hardware-Software Interaction

The computer is responsible for:

Camera

ML

Tracking

Counting

Analysis

Decision

Database

Web

Arduino is responsible for:

Receive Command

↓

Control LEDs

↓

Execute Signal Timing

This separation is important because Arduino does not have the computational resources required for the machine-learning component.

23. Prototype Layout

The physical prototype can represent a four-way intersection:

NORTH

│

🚗 🚗 🚗

│

│

─────────────┼────────────

🚗           │          🚗

WEST         │          EAST

🚗           │          🚗

─────────────┼────────────

│

│

🚗 🚗 🚗

│

SOUTH

Traffic-light LEDs will be positioned at the intersection.

The camera will be positioned so that the software can observe the lanes.

24. Testing Design

Testing will be divided into several levels.

24.1 Unit Testing

Individual components will be tested independently.

Examples:

- Camera connection.

- Vehicle detection.

- Vehicle counting.

- Green-time calculation.

- Arduino communication.

24.2 Integration Testing

Components will be tested together.

Example:

Camera

↓

ML

↓

Counting

↓

Adaptive Algorithm

↓

Arduino

24.3 System Testing

The complete system will be tested from camera input to traffic-light output.

24.4 Performance Testing

The adaptive system will be compared with a fixed-time system.

Metrics:

Average waiting time

Queue length

Vehicles cleared

Detection accuracy

Counting accuracy

25. Example Test Case

Test Case: High Traffic Lane

Input:

North = 5

South = 18

East = 4

West = 7

Expected result:

Priority = South

With:

Base = 20

Additional = 2

Expected green time:

20 + (18 × 2)

= 56 seconds

Arduino should receive:

SOUTH,56

Expected traffic-light behavior:

South → GREEN

North → RED

East  → RED

West  → RED

26. Design Constraints

The design will follow these constraints:

- One intersection only.

- Prototype traffic lights only.

- One main camera setup.

- Lightweight database.

- Simple adaptive algorithm.

- Pre-trained ML model initially.

- Arduino handles physical signal control.

- Machine learning runs on a computer.

- Web dashboard provides monitoring rather than controlling public traffic infrastructure.

27. Project Folder Structure

A possible implementation structure is:

adaptive-traffic-system/

│

├── backend/

│   ├── app.py

│   ├── config.py

│   │

│   ├── ml/

│   │   ├── detector.py

│   │   ├── tracker.py

│   │   └── counter.py

│   │

│   ├── traffic/

│   │   ├── analyzer.py

│   │   └── controller.py

│   │

│   ├── hardware/

│   │   └── arduino.py

│   │

│   ├── database/

│   │   ├── database.py

│   │   └── models.py

│   │

│   └── routes/

│       ├── traffic.py

│       ├── signal.py

│       └── system.py

│

├── frontend/

│   ├── index.html

│   ├── css/

│   │   └── style.css

│   └── js/

│       └── dashboard.js

│

├── arduino/

│   └── traffic_controller.ino

│

├── models/

│   └── vehicle_model/

│

├── data/

│   └── traffic.db

│

└── README.md

28. Final System Architecture

The complete proposed architecture is:

┌──────────────┐

│    CAMERA    │

└──────┬───────┘

│

▼

┌───────────────────┐

│ VIDEO PROCESSING  │

└─────────┬─────────┘

│

▼

┌───────────────────┐

│ ML VEHICLE        │

│ DETECTION         │

└─────────┬─────────┘

│

▼

┌───────────────────┐

│ VEHICLE TRACKING  │

└─────────┬─────────┘

│

▼

┌───────────────────┐

│ LANE ASSIGNMENT   │

└─────────┬─────────┘

│

▼

┌───────────────────┐

│ VEHICLE COUNTING  │

└─────────┬─────────┘

│

▼

┌───────────────────┐

│ TRAFFIC ANALYSIS  │

└─────────┬─────────┘

│

▼

┌───────────────────┐

│ ADAPTIVE SIGNAL   │

│ CONTROLLER        │

└───────┬─┬─────────┘

│ │

┌─────────────┘ └──────────────┐

│                              │

▼                              ▼

┌─────────────────┐             ┌───────────────┐

│     ARDUINO     │             │   DATABASE    │

└────────┬────────┘             └───────┬───────┘

│                              │

▼                              │

┌─────────────────┐                     │

│ TRAFFIC LIGHTS  │                     │

│      LEDs       │                     │

└─────────────────┘                     │

│

▼

┌─────────────────┐

│  WEB BACKEND    │

└────────┬────────┘

│

▼

┌─────────────────┐

│ WEB DASHBOARD   │

└─────────────────┘

29. Conclusion

The Software Design Document provides the technical design for the Machine Learning-Based Adaptive Traffic Signal System.

The design intentionally keeps the project manageable by focusing on a single four-way prototype intersection.

The most important design principle is that machine learning handles traffic observation, while a simple adaptive algorithm handles traffic-signal decisions.

The system follows:

Camera → ML Detection → Tracking → Counting → Traffic Analysis → Adaptive Decision → Arduino → Traffic Lights

while the web application provides:

Monitoring → Data Storage → Visualization → Historical Analysis

This architecture provides a clear separation between the AI component, traffic-control logic, hardware component, database, and web application, making the system easier to develop, test, and extend in the future.