SOFTWARE REQUIREMENTS SPECIFICATION (SRS)

Project Title

Machine Learning-Based Adaptive Traffic Signal System

Document: Software Requirements Specification
Version: 1.0

1. Introduction

1.1 Purpose

This Software Requirements Specification (SRS) defines the functional and non-functional requirements of the Machine Learning-Based Adaptive Traffic Signal System.

The system is a prototype designed to use cameras and machine learning to detect and count vehicles approaching a four-way intersection. The detected traffic information will be analyzed to determine traffic demand, and the system will dynamically adjust the green-light duration for the lane requiring greater priority.

An Arduino microcontroller will control the prototype traffic lights based on decisions generated by the software.

A web-based dashboard will allow an authorized user to monitor traffic conditions, vehicle counts, signal status, and system performance.

1.2 Scope

The proposed system will focus on one four-way intersection.

The system will consist of:

- Camera-based traffic monitoring.

- Machine-learning-based vehicle detection.

- Vehicle tracking.

- Vehicle counting.

- Lane-based traffic analysis.

- Adaptive traffic-signal decision-making.

- Arduino-based traffic-light control.

- Web-based monitoring.

- Database storage.

- System performance evaluation.

The system is intended as an academic prototype and will not directly control traffic lights on public roads.

1.3 Intended Audience

This document is intended for:

- Project developers.

- Project supervisors.

- Students.

- System testers.

- System administrators/operators.

- Researchers interested in intelligent traffic management.

1.4 Definitions and Acronyms

| Term | Definition |
| --- | --- |
| ML | Machine Learning |
| AI | Artificial Intelligence |
| CV | Computer Vision |
| Camera | Device used to capture traffic video |
| Vehicle Detection | Identifying vehicles in an image/video frame |
| Vehicle Tracking | Following a detected vehicle across multiple frames |
| Lane | A designated traffic approach/direction |
| Traffic Demand | Amount of traffic requiring service at a lane |
| Adaptive Signal | Traffic signal whose timing changes according to traffic conditions |
| Arduino | Microcontroller used to control prototype traffic lights |
| Dashboard | Web interface used to monitor the system |
| Green Time | Duration for which a traffic direction receives a green signal |
| Fixed-Time Signal | Traffic signal using predetermined timing |

2. Overall Description

2.1 Product Perspective

The system will operate as an integrated software and hardware solution.

The overall system is:

CAMERA

│

▼

┌──────────────────┐

│ Machine Learning │

│ Vehicle Detection│

└────────┬─────────┘

│

▼

┌──────────────────┐

│ Vehicle Tracking │

└────────┬─────────┘

│

▼

┌──────────────────┐

│ Lane Vehicle     │

│ Counting         │

└────────┬─────────┘

│

▼

┌──────────────────┐

│ Traffic Analysis │

└────────┬─────────┘

│

▼

┌──────────────────┐

│ Adaptive Signal  │

│ Decision         │

└───────┬──────────┘

│

┌───────┴────────┐

▼                ▼

┌─────────┐     ┌──────────────┐

│ Arduino │     │ Web Dashboard│

└────┬────┘     └──────────────┘

│

▼

Traffic Light

LEDs

2.2 Product Functions

The system shall provide the following major functions:

Traffic Monitoring

Capture video from a camera.

Vehicle Detection

Detect vehicles appearing in captured frames.

Vehicle Tracking

Track detected vehicles across frames.

Vehicle Counting

Determine the number of vehicles in each lane.

Traffic Analysis

Determine the relative traffic demand of each lane.

Adaptive Control

Select a priority lane and determine an appropriate green-light duration.

Hardware Control

Send the decision to Arduino.

Web Monitoring

Display traffic and signal information through a web interface.

Data Storage

Store traffic and signal information.

Performance Evaluation

Provide data that can be used to compare adaptive and fixed-time signal control.

2.3 User Classes

The prototype will have one primary user.

System Operator

The operator will be able to:

- Access the dashboard.

- View traffic conditions.

- View vehicle counts.

- View current signal status.

- View system status.

- View historical traffic information.

- Start or stop the system.

- Configure selected system parameters if authorized.

The operator does not manually count vehicles or manually determine the priority lane during normal operation.

2.4 Operating Environment

The system will operate on a computer connected to:

- Camera.

- Arduino.

- Prototype traffic lights.

Software environment

The proposed environment includes:

Operating System

↓

Python

↓

ML + OpenCV

↓

Backend

↓

Database

↓

Web Dashboard

Arduino will communicate with the computer through USB/serial communication.

2.5 Design Constraints

The following constraints apply:

- The system will initially support one intersection.

- The system will use predefined lane regions.

- Arduino will be used for prototype signal control.

- Machine learning will run on the computer rather than Arduino.

- The system will not directly control public-road traffic infrastructure.

- The first version will use a simple adaptive algorithm.

- The project should remain achievable within an academic project timeframe.

- Camera quality and computer processing capacity may affect ML performance.

3. Functional Requirements

3.1 Camera Management Requirements

FR-01: Camera Initialization

The system shall initialize the configured camera when the traffic-control system starts.

FR-02: Camera Connection

The system shall determine whether the camera is available.

FR-03: Video Capture

The system shall continuously capture video frames from the camera while the system is active.

FR-04: Camera Error

If the camera becomes unavailable, the system shall display an appropriate error or warning.

Example:

Camera: DISCONNECTED

3.2 Vehicle Detection Requirements

FR-05: Vehicle Detection

The system shall process captured frames using a machine-learning-based vehicle detection model.

FR-06: Vehicle Identification

The system shall identify supported vehicle objects in the video.

Possible categories include:

- Car

- Bus

- Truck

- Motorcycle

The exact supported categories will depend on the selected ML model.

FR-07: Detection Confidence

The system shall use a configurable confidence threshold to determine whether a detected object should be considered a valid vehicle.

FR-08: Detection Output

The system shall generate detection information containing, where supported:

Vehicle class

Confidence

Position

Bounding box

3.3 Vehicle Tracking Requirements

FR-09: Vehicle Tracking

The system shall track detected vehicles across consecutive video frames.

FR-10: Vehicle Identification

The tracking system shall assign a temporary identifier to detected vehicles.

Example:

Vehicle ID: 001

Vehicle ID: 002

Vehicle ID: 003

FR-11: Duplicate Prevention

The system shall prevent the same vehicle from being counted repeatedly merely because it appears in multiple frames.

3.4 Lane Management Requirements

FR-12: Lane Definition

The system shall support predefined regions representing the traffic lanes or approaches.

Example:

North

South

East

West

FR-13: Lane Assignment

The system shall determine the lane associated with each detected vehicle based on its position.

FR-14: Lane Count

The system shall calculate the current number of detected vehicles associated with each lane.

Example:

North = 8

South = 15

East  = 4

West  = 7

3.5 Traffic Analysis Requirements

FR-15: Traffic Demand Calculation

The system shall analyze vehicle counts to determine traffic demand.

FR-16: Traffic Classification

The system may classify traffic into categories such as:

LOW

MEDIUM

HIGH

For example:

0–5     → LOW

6–10    → MEDIUM

11+     → HIGH

The thresholds should be configurable during development.

FR-17: Priority Determination

The system shall identify the lane with the greatest traffic demand.

Example:

North = 8

South = 15

East = 4

West = 7

Priority = South

3.6 Adaptive Signal Requirements

FR-18: Green-Time Calculation

The system shall calculate green-light duration based on traffic demand.

The initial formula shall be:

Green Time =

Base Time + (Vehicle Count × Additional Time)

FR-19: Minimum Green Time

The system shall not allocate less than the configured minimum green-light duration.

Example:

Minimum = 20 seconds

FR-20: Maximum Green Time

The system shall not allocate more than the configured maximum green-light duration.

Example:

Maximum = 60 seconds

FR-21: Priority Signal

The system shall assign the calculated green-light duration to the selected priority lane.

FR-22: Signal Sequence

The system shall implement a safe prototype signal sequence:

GREEN

↓

YELLOW

↓

RED

FR-23: Fairness

The system should prevent a lane from being continuously ignored when other lanes repeatedly have higher vehicle counts.

A simple waiting-time or waiting-cycle mechanism may be used.

3.7 Arduino Requirements

FR-24: Arduino Connection

The system shall establish communication with the Arduino.

FR-25: Signal Command

The software shall send traffic-signal commands to Arduino.

Example:

SOUTH,48

FR-26: Arduino Interpretation

Arduino shall interpret the received command and control the corresponding LEDs.

FR-27: Traffic-Light Control

Arduino shall control the prototype:

Red LED

Yellow LED

Green LED

for each direction.

FR-28: Arduino Error

The system shall detect and report Arduino communication failure where possible.

3.8 Web Dashboard Requirements

FR-29: Dashboard Access

The operator shall be able to access the system through a web browser.

FR-30: Vehicle Count Display

The dashboard shall display the current vehicle count for each lane.

Example:

North: 8

South: 15

East: 4

West: 7

FR-31: Traffic Level Display

The dashboard shall display the traffic level for each lane.

FR-32: Priority Display

The dashboard shall display the current priority lane.

FR-33: Signal Display

The dashboard shall display the current signal state.

Example:

South → GREEN

FR-34: Green-Time Display

The dashboard shall display the allocated green-light duration.

FR-35: Remaining-Time Display

Where supported, the dashboard shall display the remaining signal time.

Example:

South Green

Remaining: 32 seconds

FR-36: Component Status

The dashboard shall display system-component status.

Example:

Camera: Connected

ML: Running

Arduino: Connected

Database: Connected

3.9 Database Requirements

FR-37: Traffic Record Storage

The system shall store traffic observations.

Each record may contain:

ID

Timestamp

Lane

Vehicle Count

Traffic Level

FR-38: Signal Record Storage

The system shall store signal decisions.

Each record may contain:

ID

Timestamp

Priority Lane

Green Time

FR-39: System Event Storage

The system shall store important system events and errors.

FR-40: Historical Data

The operator shall be able to view historical traffic information through the dashboard.

4. Non-Functional Requirements

4.1 Performance Requirements

NFR-01: Processing

The system should process camera frames sufficiently quickly to provide near-real-time traffic information for the prototype.

NFR-02: Response

The system should update traffic information without excessive delay.

NFR-03: Dashboard

The dashboard should display updated traffic information within a reasonable period after new analysis results are generated.

4.2 Reliability Requirements

NFR-04: Error Detection

The system shall detect major component failures where possible.

NFR-05: Safe Signal Behavior

The system shall prevent conflicting green signals in the prototype.

NFR-06: Recovery

The system should be capable of recovering from temporary software or communication failures where practical.

4.3 Usability Requirements

NFR-07: Simple Interface

The dashboard shall use a simple and understandable interface.

NFR-08: Clear Information

Traffic information shall be presented clearly.

For example:

┌─────────────────────────────┐

│ SOUTH                       │

│ 15 Vehicles                │

│ HIGH TRAFFIC               │

│ GREEN                      │

│ 32 seconds remaining       │

└─────────────────────────────┘

4.4 Maintainability Requirements

NFR-09: Modular Design

The software shall be divided into independent modules.

Recommended modules:

Camera

ML Detection

Tracking

Counting

Traffic Analysis

Adaptive Control

Arduino

Database

Web

NFR-10: Configuration

Important parameters should be configurable without modifying large portions of the source code.

Examples:

Minimum green time

Maximum green time

Additional time per vehicle

Detection confidence

4.5 Scalability Requirements

NFR-11: Future Expansion

The architecture should allow the system to be extended to additional intersections in future versions.

The first implementation will remain limited to one intersection.

4.6 Security Requirements

NFR-12: Authentication

The dashboard should require authentication if administrative functions are enabled.

NFR-13: Password Security

Passwords shall not be stored as plain text.

NFR-14: Backend Protection

Database credentials and system configuration shall not be exposed through the frontend.

NFR-15: Access Control

Only authorized users should be able to change important system parameters.

5. External Interface Requirements

5.1 Camera Interface

The camera will provide:

Video Stream

↓

Computer

The software will access the camera using a supported camera interface.

5.2 Arduino Interface

The computer and Arduino will communicate using:

USB

↓

Serial Communication

Example command:

NORTH,35

5.3 User Interface

The operator will access the system through a web browser.

The dashboard should contain:

Dashboard

├── Live Traffic

├── Signal Status

├── Traffic History

└── System Status

5.4 Database Interface

The backend will communicate with the database to:

- Insert traffic records.

- Retrieve historical records.

- Store signal events.

- Store system events.

6. System Use Cases

6.1 Use-Case Diagram Description

The main actor is:

System Operator

The main use cases are:

┌─────────────────────────┐

│ Machine Learning-Based   │

│ Adaptive Traffic Signal  │

│ System                  │

│                         │

Operator ──────────►│ View Dashboard          │

Operator ──────────►│ View Traffic            │

Operator ──────────►│ View Signal Status      │

Operator ──────────►│ View History            │

Operator ──────────►│ Manage System           │

│                         │

Camera ────────────►│ Detect Vehicles         │

│                         │

Arduino ◄──────────►│ Control Traffic Lights  │

└─────────────────────────┘

7. Detailed Use Cases

UC-01: Monitor Traffic

Actor: System Operator

Description

The operator views the current traffic conditions.

Preconditions

- System is running.

- Camera is connected.

- ML system is running.

Main Flow

- Operator opens dashboard.

- System retrieves current traffic information.

- Dashboard displays vehicle counts.

- Dashboard displays traffic levels.

- Dashboard displays priority lane.

Postcondition

The operator can see the current traffic situation.

7.2 UC-02: Detect Vehicles

Actor: Camera / System

Main Flow

- Camera captures a frame.

- ML model processes the frame.

- Vehicles are detected.

- Detection information is passed to the tracking module.

Postcondition

Detected vehicles are available for tracking.

7.3 UC-03: Count Vehicles

Actor: System

Main Flow

- Receive vehicle detections.

- Track vehicles.

- Determine lane for each vehicle.

- Update lane counts.

- Store traffic information.

Postcondition

Current vehicle counts are available.

7.4 UC-04: Determine Priority Lane

Actor: System

Main Flow

- Obtain lane vehicle counts.

- Compare traffic demand.

- Identify the lane requiring priority.

- Calculate green time.

Example

North = 5

South = 15

East = 4

West = 8

Result:

Priority = South

7.5 UC-05: Control Traffic Light

Actor: Arduino

Main Flow

- Receive priority lane.

- Receive green-light duration.

- Activate appropriate green LED.

- Keep other conflicting directions red.

- Change green to yellow.

- Change yellow to red.

- Wait for the next decision.

7.6 UC-06: View Historical Traffic

Actor: System Operator

Main Flow

- Operator opens Traffic History.

- System retrieves stored traffic records.

- System displays traffic information.

- Operator analyzes traffic behavior.

7.7 UC-07: View System Status

Actor: System Operator

The dashboard shall show:

Camera       → Connected

ML Model     → Running

Arduino      → Connected

Database     → Connected

System       → Active

8. System Data Requirements

8.1 Traffic Data

The system will process:

Vehicle ID

Vehicle Type

Lane

Timestamp

Detection Confidence

8.2 Decision Data

The adaptive controller will use:

Vehicle Count

Traffic Level

Waiting Information

Priority Lane

Green Time

9. Business Rules

The system will follow these rules.

BR-01

The lane with the highest traffic demand should normally receive priority.

BR-02

Green time shall not fall below the configured minimum.

BR-03

Green time shall not exceed the configured maximum.

BR-04

The same vehicle shall not be repeatedly counted because of consecutive frames.

BR-05

Conflicting traffic directions shall not receive green signals simultaneously.

BR-06

A lane should not be ignored indefinitely.

BR-07

Traffic decisions should be based on automatically detected traffic information rather than manual vehicle entry.

10. Adaptive Timing Example

Suppose the system detects:

North = 5

South = 18

East  = 4

West  = 7

The highest traffic is:

South = 18

Using:

Base Time = 20

Additional Time = 2

the system calculates:

20 + (18 × 2)

= 56 seconds

Therefore:

South → GREEN → 56 seconds

The dashboard displays:

Priority: SOUTH

Traffic: HIGH

Green Time: 56 seconds

Arduino receives:

SOUTH,56

11. System States

The system shall have several states.

START

│

▼

INITIALIZING

│

▼

READY

│

▼

RUNNING

/       \

/         \

▼           ▼

ERROR STATE     STOPPED

│

▼

RECOVERY

│

▼

RUNNING

12. Error and Exception Requirements

Camera Failure

Camera disconnected

↓

System detects failure

↓

Dashboard displays warning

↓

System attempts recovery

Arduino Failure

Arduino disconnected

↓

System displays warning

↓

Signal commands are suspended

ML Failure

ML processing failure

↓

System logs error

↓

Dashboard reports ML status

Database Failure

The system should report the database problem and, where practical, continue the traffic-processing portion without losing system safety.

13. Data Flow

The main data flow is:

Camera

│

▼

Video Frame

│

▼

ML Detector

│

▼

Vehicle Detections

│

▼

Vehicle Tracker

│

▼

Tracked Vehicles

│

▼

Lane Counter

│

▼

Vehicle Counts

│

▼

Traffic Analyzer

│

▼

Traffic Demand

│

▼

Adaptive Controller

│

├──────────────► Database

│

├──────────────► Web Dashboard

│

▼

Arduino

│

▼

Traffic Lights

14. Acceptance Criteria

The project will be considered successful if the prototype can demonstrate the following:

| Requirement | Acceptance Condition |
| --- | --- |
| Camera | Camera successfully provides video |
| ML | Vehicles are detected from video |
| Tracking | Same vehicle is not repeatedly counted |
| Counting | Vehicles are assigned to appropriate lanes |
| Analysis | Traffic demand is calculated |
| Priority | Highest-demand lane is identified |
| Timing | Green time changes according to demand |
| Arduino | Arduino receives control commands |
| Traffic lights | LEDs respond correctly |
| Dashboard | Current traffic is displayed |
| Database | Traffic records are stored |
| Evaluation | Adaptive and fixed-time systems are compared |

15. Recommended Minimum Viable System

To prevent the project from becoming too complex, the first working version should contain only:

CAMERA

↓

Vehicle Detection

↓

Vehicle Count

↓

Lane Comparison

↓

Priority Selection

↓

Green-Time Formula

↓

Arduino

↓

Traffic LEDs

↓

Web Dashboard

You do not need to implement advanced AI features before this basic pipeline works.

The most important part of the project is proving:

When traffic conditions change, the system can detect that change and automatically change the traffic-signal timing.

16. SRS Conclusion

The Machine Learning-Based Adaptive Traffic Signal System is designed as a manageable prototype that combines machine learning, computer vision, embedded systems, and web technologies.

The system will automatically obtain traffic information from cameras rather than depending on manual data entry. Machine learning will detect vehicles, tracking will prevent duplicate counting, and lane analysis will determine traffic demand. An adaptive controller will then use this information to determine the appropriate traffic-signal timing.

Arduino will execute the decision using prototype traffic-light LEDs, while the web dashboard will provide real-time monitoring and historical analysis.

The core requirements can therefore be summarized as:

OBSERVE

↓

DETECT

↓

TRACK

↓

COUNT

↓

ANALYZE

↓

DECIDE

↓

CONTROL

↓

MONITOR

↓

REPEAT

The SRS establishes the requirements that will guide the next document: the Software Design Document (SDD), where these requirements will be translated into the system architecture, database design, modules, UML diagrams, interfaces, algorithms, and detailed implementation structure.